Layers of OSI Model

Last Updated : 8 Apr, 2026

The OSI Model is a conceptual framework created by the International Organization for Standardization (ISO) to describe how data is transmitted across a network using a structured seven-layer architecture.

- Divides network communication into seven functional layers.

- Assigns specific responsibilities to each layer.

- Promotes compatibility between different networking systems.

- Simplifies network design, implementation, and troubleshooting.

Layer 1: Physical Layer

This is the lowest layer of the OSI reference model and responsible for the actual physical connection between the devices.

- Physical Layer is responsible for transmitting individual bits from one node to the next.

- When receiving data, this layer will get the signal received and convert it into 0s and 1s and send them to the Data Link layer.

- Common physical layer devices are Hub, Repeater, Modem, and Cables.

Physical Layer

Functions

- Bit Synchronization: This layer provides the synchronization of the bits by providing a clock and that clock controls both sender and receiver, thus providing synchronization at the bit level.

- Bit Rate Control: This layer defines the transmission rate i.e. the number of bits sent per second.

- Physical Topologies: Specifies how the different, devices/nodes are arranged in a network i.e. bus topology, star topology, or mesh topology.

- Transmission Mode: Defines how data flows between the two connected devices and the various transmission modes possible are Simplex, half-duplex and full duplex.

Layer 2: Data Link Layer (DLL)

The data link layer is responsible for the node-to-node delivery of the message and make sure data transfer is error-free from one node to another, over the physical layer.

- When a packet arrives in a network, it is the responsibility of the DLL to transmit it to the host using its MAC address.

- Packet in the DLL is referred to as Frame and Switches and Bridges are common DLL devices.

- The packet received from the Network layer is further divided into frames depending on the frame size of the NIC (Network Interface Card).

- DLL also encapsulates Sender and Receiver’s MAC address in the header.

Sublayers of Data Link Layer

- Logical Link Control (LLC): Manages communication between upper layers and performs error and flow control.

- Media Access Control (MAC): Controls how devices gain access to the transmission medium and use MAC addressing.

Functions

- Framing: Framing is a function of the Data Link Layer that divides data into frames by adding start and end bits so the receiver can identify each unit of data.

- Physical Addressing: After creating frames, the Data Link Layer adds MAC addresses of the sender and receiver in the header to ensure correct delivery.

- Error Control: The Data Link Layer detects and retransmits damaged or lost frames to ensure reliable communication.

- Flow Control: Flow control manages the data transmission rate so the sender does not overwhelm the receiver.

- Access Control: The MAC sublayer decides which device can transmit data when multiple devices share the same communication channel.

Layer 3: Network Layer

The Network Layer works for the transmission of data from one host to the other located in different networks and also takes care of packet routing.

- The sender and receiver's IP address are placed in the header by the network layer.

- Segment in the Network layer are referred to as Packets.

- Network layer is implemented by networking devices such as routers and switches.

Functions

- Routing: The network layer protocols determine which route is suitable from source to destination.

- Logical Addressing: The Network Layer assigns unique IP addresses to sender and receiver so devices can be identified across different networks.

Layer 4: Transport Layer

This layer is responsible for end-to-end delivery of the message and Transport Layer provides services to the application layer and takes services from the network layer.

- The data in the transport layer is referred to as Segments and it is responsible for successful data transmission.

- Protocols used in Transport Layer are TCP, UDP NetBIOS, PPTP.

- It adds source and destination port number in its header and forwards the segmented data to the Network Layer.

- Example: When a web application requests a web server, it typically uses port number 80, because this is the default port assigned to web applications.

Functions

- Segmentation and Reassembly: The Transport Layer divides the message into smaller segments and reassembles them at the destination.

- Service Point Addressing: The Transport Layer uses port numbers to ensure the message is delivered to the correct process on the destination device.

Services

- Connection-Oriented Service: Establishes a connection before data transfer and ensures reliable delivery with error checking and acknowledgments.

- Connectionless Service: Sends data without establishing a connection, providing faster but less reliable communication.

Layer 5: Session Layer

Session Layer is responsible for the establishment of connections, management of connections, terminations of sessions between two devices and provides authentication and security.

Functions

- Session Establishment, Maintenance, and Termination: The layer allows the two processes to establish, use, and terminate a connection.

- Synchronization: This layer allows a process to add checkpoints that are considered synchronization points in the data.

- These synchronization points help to identify the error so that the data is re-synchronized properly

- Dialog Controller: The session layer allows two systems to start communication with each other in half-duplex or full duplex.

Example

Let us consider a scenario where a user wants to send a message through some Messenger application running in their browser. The “Messenger” here acts as the application layer which provides the user with an interface to create the data. This message or so-called Data is compressed, optionally encrypted (if the data is sensitive), and converted into bits (0’s and 1’s) so that it can be transmitted.

Layer 6: Presentation Layer

The Presentation Layer is also called the Translation layer and data from the application layer is extracted here and manipulated as per the required format to transmit over the network.

- Protocols used in the Presentation Layer are TLS/SSL (Transport Layer Security / Secure Sockets Layer).

- JPEG, MPEG, GIF, are standards or formats used for encoding data, which is part of the presentation layer’s role.

Functions

- Encryption/ Decryption: Data encryption translates the data into another form or code. The encrypted data is known as the ciphertext, and the decrypted data is known as plain text.

- A key value is used for encrypting as well as decrypting data.

- Compression: Reduces the number of bits that need to be transmitted on the network.

Layer 7: Application Layer

At the very top of the OSI Reference Model stack of layers, we find the Application Layer which is implemented by the network applications. These applications produce the data to be transferred over the network.

- This layer also serves as a window for the application services to access the network and for displaying information.

- Protocols used in the Application layer are SMTP, FTP, DNS, etc.

Application Layer

Functions

- Network Virtual Terminal (NVT): It allows a user to log on to a remote host.

- File Transfer Access and Management (FTAM): This application allows a user to access, retrieve and manage or control files from a remote computer. .

- Directory Services: This application provides distributed database sources and access for global information about various objects and services.

Data Flows in the OSI Model

When we transfer information from one device to another, it travels through 7 layers of OSI model. First data travels down through 7 layers from the sender's end and then climbs back 7 layers on the receiver's end. Data flows through the OSI model in a step-by-step process:

- Application Layer: Applications create the data.

- Presentation Layer: Data is formatted and encrypted.

- Session Layer: Connections are established and managed.

- Transport Layer: Data is broken into segments for reliable delivery.

- Network Layer: Segments are packaged into packets and routed.

- Data Link Layer: Packets are framed and sent to the next device.

- Physical Layer: Frames are converted into bits and transmitted physically.

Each layer adds specific information to ensure the data reaches its destination correctly, and these steps are reversed upon arrival.

Example

We can understand how data flows through OSI Model with the help of an example mentioned below. Let us suppose, Person A sends an e-mail to his friend Person B.

- Step 1: Person A interacts with e-mail application like Gmail, outlook, etc. Writes his email to send. (This happens at Application Layer).

- Step 2: At Presentation Layer, Mail application prepares for data transmission like encrypting data and formatting it for transmission.

- Step 3: At Session Layer, there is a connection established between the sender and receiver on the internet.

- Step 4: At Transport Layer, Email data is broken into smaller segments. It adds sequence number and error-checking information to maintain the reliability of the information.

- Step 5: At Network Layer, addressing of packets is done in order to find the best route for transfer.

- Step 6: At Data Link Layer, data packets are encapsulated into frames, then MAC address is added for local devices and then it checks for error using error detection.

- Step 7: At Physical Layer, Frames are transmitted in the form of electrical/ optical signals over a physical network medium like ethernet cable or WiFi.

After the email reaches the receiver i.e. Person B, the process will reverse and decrypt the e-mail content. At last, the email will be shown on Person B email client.

OSI Layers

Protocols Used in the OSI Layers

| Layer | Working | Protocol Data Unit | Protocols |
| --- | --- | --- | --- |
| Physical Layer | Establishing Physical Connections between Devices. | Bits | USB, SONET/SDH |
| DataLink Layer | Node to Node Delivery of Message. | Frames | Ethernet, PPP |
| Network Layer | Transmission of data from one host to another, located in different networks. | Packets | IP, ICMP, IGMP, OSPF |
| Transport Layer | Take Service from Network Layer and provide it to the Application Layer. | Segments (for TCP) or Datagrams (for UDP) | TCP, UDP, SCTP |
| Session Layer | Establishes Connection, Maintenance, Ensures Authentication and Ensures security. | Data | NetBIOS, RPC, PPTP |
| Presentation Layer | Data from the application layer is extracted and manipulated in the required format for transmission. | Data | TLS/SSL, MIME |
| Application Layer | Helps in identifying the client and synchronizing communication. | Data | FTP, SMTP, DNS, DHCP |

OSI and TCP IP Model (Part 1)

Visit Course

Suggested Quiz

7 Questions

Which OSI layer is responsible for bit rate control and transmission mode (simplex, half-duplex, full-duplex)?

- A

Data Link Layer

- B

Network Layer

- C

Transport Layer

- D

Physical Layer

In the OSI model, error control is implemented at:

- A

Data Link Layer and Transport Layer

- B

Physical Layer and Network Layer

- C

Application Layer only

- D

Session Layer

At the Data Link Layer, what is the main role of the MAC sublayer?

- A

Address translation between IP and MAC

- B

Error detection only

- C

Controlling access to the shared communication medium

- D

Assigning logical addresses

Which device operates at both the Data Link Layer and Network Layer?

- A

Switch

- B

Router

- C

Gateway

- D

Hub

In TCP/IP, which layer corresponds to the combination of Session, Presentation, and Application layers of OSI?

- A

Application Layer

- B

Transport Layer

- C

Internet Layer

- D

Network Access Layer

Which of the following is NOT a function of the Transport Layer?

- A

Segmentation and Reassembly

- B

Service Point Addressing

- C

Routing to destination

- D

Flow and Error Control

What does the Session Layer mainly provide?

- A

Encryption and Compression

- B

Dialog control and synchronization

- C

Address resolution

- D

Routing decision

Quiz Completed Successfully

Your Score :0/7

Accuracy :0%

Show More

Login to View Explanation

1/7

< PreviousNext >

Comment

K

Kundana Thiyari

1.35k

Article Tags:

Computer Networks

Why is Computer Networking Important?

Computer networks are important because they:

- Allow fast communication (like emails, video calls, and chats)

- Help in sharing resources (like files, software, internet, printers)

- Make data storage and backup easier with centralized systems

- Enable remote access and teamwork from different locations

Importance in Interview

In technical interviews, Computer Networking is a core topic because:

- It’s used in almost every job role — from developers to system admins.

- Companies want candidates who understand how devices talk to each other.

- Real-world scenarios (like “What happens when you type www.google.com?”) often test your practical networking knowledge.

- For companies like TCS, Infosys, Wipro, Cognizant, Capgemini, Computer Networks is a frequently asked topic in both written tests and face-to-face interviews.

Note to keep in mindThis blog contains different types of questions and answers on the following topic -
Networking interview questions for freshers
OSI model questions
TCP/IP vs UDP
Layer 2 vs Layer 3 interview questions

Computer Network Interview Questions - For Freshers(Easy Level Problem)

Networking interview questions for freshers

Question 1 – What is computer network?

Ans – A computer network is a system where two or more computers and devices are connected to share data, resources, and communication.

- These devices communicate using wired (Ethernet cables) or wireless (Wi-Fi, Bluetooth) connections.

- Computer networks enable file sharing, internet access, and remote communication.

- They can range from small networks, like those in homes and offices, to large-scale networks like the internet.

Types of Computer Networks:

- LAN (Local Area Network) – Connects devices within a limited area (e.g., home, office).

- WAN (Wide Area Network) – Covers large geographical areas (e.g., the internet).

- MAN (Metropolitan Area Network) – Connects multiple LANs within a city.

- PAN (Personal Area Network) – A small network for personal devices (e.g., Bluetooth).

Question 2 – What is an IP Address?

Ans – An IP address, or Internet Protocol address, is a unique identifier assigned to each device connected to a network.

- It serves two main purposes: identifying the host or network interface and providing the location of the device in the network.

- There are two versions of IP addresses: IPv4 and IPv6.

- IPv4 addresses are 32-bit numbers typically expressed in decimal format as four octets (e.g., 192.168.1.1).

- IPv6 addresses are 128-bit numbers designed to replace IPv4 due to the latter’s limited address space.

Question 3 – What are the different classes of IPv4?

Ans – IPv4 addresses are categorized into five classes: A, B, C, D, and E, based on their leading bits and intended use:

- Class A: Supports a vast number of hosts (over 16 million) and is used for large networks (e.g., government).

- Class B: Suitable for medium-sized networks, allowing up to 65,536 hosts.

- Class C: Designed for small networks with a maximum of 256 hosts.

- Class D: Reserved for multicast groups.

- Class E: Used for experimental purposes.

Note – Each class has specific ranges of addresses that help in organizing and managing networks effectively.

Question 4 – Explain different types of networks.

Ans – Networks can be classified based on their size and purpose:

- PAN (Personal Area Network): This network connects devices within a short range (e.g., Bluetooth devices).

- LAN (Local Area Network): It covers a small geographic area like a home or office.

- MAN (Metropolitan Area Network): MAN spans a city or large campus.

- WAN (Wide Area Network): This network connects multiple LANs over large distances (e.g., the Internet).

- GAN (Global Area Network): GAN encompasses worldwide connectivity through satellite links.

Question 5 – What is a VPN?

Ans –  A Virtual Private Network (VPN) creates a secure connection over the Internet between your device and another network.

- It encrypts your data and masks your IP address, making it difficult for unauthorized users to access your information.

- VPNs are commonly used by remote workers to connect securely to their company’s internal network from anywhere in the world.

- They also help bypass geographic restrictions on content by allowing users to appear as if they are accessing the Internet from a different location.

OSI model questions

Question 6 – What is the OSI Model?

Ans – The OSI (Open Systems Interconnection) Model is a conceptual framework used to understand and implement networking protocols in seven layers:

- Layer 1: Physical Layer: Manages hardware connections.

- Layer 2: Data Link Layer: Handles error detection and correction from physical layer transmission.

- Layer 3: Network Layer: Responsible for routing packets across networks.

- Layer 4: Transport Layer: Ensures complete data transfer through error recovery and flow control.

- Layer 5: Session Layer: Manages sessions between applications.

- Layer 6: Presentation Layer: Translates data formats between systems.

- Layer 7: Application Layer: Interfaces directly with end-user applications.

Each layer serves a specific function and interacts with the layers above and below it, providing a structured approach to networking.

Question 7 – What is TCP/IP?

Ans – TCP/IP (Transmission Control Protocol/Internet Protocol) is a set of communication protocols used for interconnecting network devices on the Internet. It consists of four layers:

- Link Layer: Manages physical connections.

- Internet Layer: Handles packet routing across networks.

- Transport Layer: Ensures reliable data transfer between devices.

- Application Layer: Contains protocols for specific applications like HTTP, FTP, etc.

TCP/IP is fundamental to Internet communication, enabling diverse systems to communicate seamlessly.

Question 8 – What is DNS?

Ans – The Domain Name System (DNS) translates human-readable domain names (like www.example.com) into IP addresses that computers use to identify each other on the network.

- This system allows users to access websites using easy-to-remember names instead of numerical IP addresses.

- DNS operates through a distributed database structure that ensures quick resolution of domain names across the Internet.

Question 9 – What is DHCP?

Ans – The Dynamic Host Configuration Protocol (DHCP) automates the assignment of IP addresses within a network.

- When a device connects to the network, DHCP assigns it an available IP address from a predefined range along with other necessary configuration details like subnet mask and default gateway.

- This process simplifies network management by reducing manual configuration efforts.

Question 10 – What is NAT?

Ans –  Network Address Translation (NAT) is a method used by routers to translate private IP addresses within a local network into public IP addresses before they access the Internet.

- This allows multiple devices on a local network to share a single public IP address while keeping their internal IP addresses hidden from external networks, enhancing security and conserving IP address space.

Question 11 – What is subnetting?

Ans – Subnetting is the practice of dividing a larger network into smaller subnetworks or subnets.

- This improves routing efficiency and enhances security by isolating segments of the network from each other.

- Subnetting involves borrowing bits from the host portion of an IP address to create additional subnetworks, allowing organizations to manage their resources more effectively.

Question 12 – What are firewalls?

Ans – Firewalls are security devices or software applications that monitor incoming and outgoing traffic on a network based on predetermined security rules.

- They act as barriers between trusted internal networks and untrusted external networks, helping prevent unauthorized access while allowing legitimate traffic through.

- Firewalls can be hardware-based or software-based and are essential components of any security strategy.

Question 13 – What is ICMP?

Ans – The Internet Control Message Protocol (ICMP) is used by network devices like routers to send error messages and operational information indicating success or failure when communicating with another IP address.

- For example, ICMP can notify if an unreachable destination was encountered during packet transmission or if there was an issue with packet routing.

Question 14 – What is ARP?

Ans – Address Resolution Protocol (ARP) translates an IP address into its corresponding MAC (Media Access Control) address within a local area network (LAN).

- When one device wants to communicate with another on the same LAN but knows only its IP address, it uses ARP to discover the MAC address associated with that IP address so it can send data packets correctly.

Question 15 – Differentiate between TCP and UDP protocols?

Ans –  TCP (Transmission Control Protocol) and UDP (User Datagram Protocol) are both transport layer protocols but serve different purposes:

- Connection Type: TCP is connection-oriented, meaning it establishes a connection before data transfer, while UDP is connectionless, sending data without setting up a connection.

- Reliability: TCP is reliable as it ensures data is delivered accurately and in order with error checking and retransmission. UDP is unreliable, meaning it does not guarantee data delivery or order.

- Speed:  TCP is slower due to its error-checking and acknowledgment process, whereas UDP is faster as it has minimal overhead.

- Usage: TCP is used for applications where accuracy is important, such as web browsing, emails, and file transfers. UDP is preferred for real-time applications like video streaming, online gaming, and VoIP, where speed is more critical than reliability.

In short, TCP prioritizes accuracy, while UDP focuses on speed.

Question 16 – What are unicasting, multicasting, broadcasting, and anycasting?

Ans – These terms refer to methods of sending data across networks:

- Unicasting: Sending data from one sender to one receiver.

- Multicasting: Sending data from one sender to multiple specified receivers simultaneously.

- Broadcasting: Sending data from one sender to all possible receivers in the network.

- Anycasting: Sending data from one sender to any one of several receivers that are closest in terms of routing distance.

Question 17 – What is a switch?

Ans – A switch is a networking device that connects devices within a local area network (LAN). Unlike hubs that broadcast incoming traffic to all ports, switches intelligently forward data only to specific devices based on MAC addresses, improving efficiency and reducing collisions within the network.

Question 18 – What is routing?

Ans – Routing refers to the process of selecting paths in a network along which data packets travel from source to destination. Routers use algorithms and routing tables that contain information about various routes available in the network topology to determine optimal paths for packet forwarding.

Question 19 – Explain what VLANs are?

Ans – Virtual Local Area Networks (VLANs) allow network administrators to create separate broadcast domains within a single physical switch infrastructure without requiring additional hardware.

- VLANs enhance security by isolating sensitive data traffic from general traffic while improving performance by reducing broadcast traffic across segments.

Question 20 – Define bandwidth and latency in networking terms?

Ans – Bandwidth refers to the maximum rate at which data can be transmitted over a communication channel in a given time period, usually measured in bits per second (bps).

Latency refers to the time it takes for data packets to travel from source to destination, often measured in milliseconds (ms). High bandwidth allows more data transfer simultaneously while low latency ensures quicker responses during communication.

Most Important NoteThese all Computer Network Interview Questions were asked in the Technical Interview in different companies and we shortlisted these questions by interviewing different students. Please go through it carefully.

Computer Network Interview Questions - Intermediate and Advance Level

Question 1 – What is the difference between a router and a switch?

Ans – A router and a switch are both essential devices in networking, but they serve different purposes.

- A router connects multiple networks together, directing data packets between them.

- It operates at the network layer (Layer 3) of the OSI model and can manage traffic between different IP networks. Routers use IP addresses to determine the best path for data to travel, making them crucial for connecting to the Internet.

On the other hand, a switch operates within a single network, connecting devices like computers and printers.

- It functions at the data link layer (Layer 2) and uses MAC addresses to forward data only to the intended recipient within the same network segment.

- This targeted approach reduces unnecessary traffic and increases network efficiency.

In summary, routers connect different networks, while switches connect devices within the same network.

Question 2 – What is network topology?

Ans – Network topology refers to the arrangement or layout of various elements (links, nodes, etc.) in a computer network. It defines how devices are interconnected and how they communicate with each other. There are several types of network topologies, including:

- Star Topology: All devices are connected to a central hub or switch. This setup is easy to manage and troubleshoot but can be affected if the central device fails.

- Bus Topology: All devices share a single communication line or cable. It’s simple and cost-effective but can lead to data collisions and is less reliable if the main cable fails.

- Ring Topology: Each device is connected in a circular fashion, with data traveling in one direction. This can reduce collisions but can be disrupted if any device fails.

- Mesh Topology: Every device is interconnected, providing multiple paths for data. This topology offers high redundancy but can be complex and expensive to implement.

Understanding these topologies helps in designing efficient and reliable networks based on specific needs.

Question 3 – What role does SSL play in networking?

Ans – Secure Sockets Layer (SSL) provides encryption for data transmitted over networks like the Internet, ensuring secure communications between clients and servers by preventing eavesdropping or tampering with sensitive information such as login credentials or credit card numbers during transmission.

Question 4 – What does QoS mean in networking?

Ans – Quality of Service (QoS) refers to techniques used in networking that prioritize certain types of traffic over others to ensure optimal performance for critical applications or services.

- QoS mechanisms manage bandwidth allocation, control latency, reduce packet loss, and improve overall user experience during peak usage times.

- By implementing QoS policies, organizations can ensure that high-priority applications like VoIP or video conferencing receive sufficient bandwidth even when the network experiences congestion.

Question 5 – What are common types of cables used in networking?

Ans – Various types of cables are utilized in networking based on specific requirements:

- Twisted Pair Cable: Commonly used for Ethernet connections; it consists of pairs of insulated copper wires twisted together to reduce interference.

- Coaxial Cable: Used for cable Internet connections; it has a central conductor surrounded by insulation and shielding.

- Fiber Optic Cable: Composed of thin strands of glass or plastic that transmit data as light signals; it offers high bandwidth over long distances with minimal signal loss.

Each cable type has its advantages and disadvantages regarding speed, distance, cost, and installation complexity.

Question 6 – What are common network protocols?

Ans – Various protocols govern communication over networks:

- HTTP/HTTPS: Used for transferring web pages; HTTPS includes encryption for security.

- FTP/SFTP: File Transfer Protocols used for transferring files between client and server; SFTP adds encryption.

- SMTP: Simple Mail Transfer Protocol used for sending emails.

- POP3/IMAP: Protocols for retrieving emails from servers.

Question 7 – What is load balancing?

Ans – Load balancing distributes incoming network traffic across multiple servers or resources to ensure no single server becomes overwhelmed with requests. This process enhances application availability and reliability by optimizing resource use while improving response times for users.

Load balancers can operate at various layers of the OSI model – Layer 4 (Transport Layer) or Layer 7 (Application Layer) – and can direct traffic based on various criteria such as server health status or user location.

Question 8 – What is port forwarding?

Ans – Port forwarding is a technique used in networking that directs incoming traffic from one port on your router to another port on your local network device (like a computer or server).

- This allows external devices to access services hosted on internal devices behind a router’s firewall without exposing all internal ports directly to the Internet.

- For example, if you host a web server internally on port 80 but want users outside your local network to access it through your public IP address, you would set up port forwarding rules accordingly.

Question 9 – Explain what ICMP is used for?

Ans – The Internet Control Message Protocol (ICMP) is primarily used for sending error messages and operational information about an IP operation back to the source device when issues arise during packet transmission – such as unreachable hosts or time exceeded errors during routing processes.

- ICMP plays an essential role in diagnostics tools like ping and traceroute, helping administrators troubleshoot connectivity issues by providing feedback about packet delivery status across networks.

Question 10 – What are some common security threats in networking?

Ans – Networking faces various security threats that can compromise data integrity:

- Malware: Malicious software designed to harm systems or steal sensitive information.

- Phishing: Fraudulent attempts to obtain sensitive information by masquerading as trustworthy entities.

- DDoS Attacks: Distributed Denial-of-Service attacks overwhelm systems with excessive requests.

- Man-in-the-Middle Attacks: Intercepting communications between two parties without their knowledge.

Question 11 – Explain what VoIP is?

Ans – Voice over Internet Protocol (VoIP) enables voice communication over Internet connections instead of traditional telephone lines by converting voice signals into digital packets transmitted over IP networks.

- VoIP technology allows users to make phone calls using broadband connections via applications like Skype or Zoom while offering features such as video conferencing at lower costs compared to conventional telephony services.

Question 12 – What does RDP stand for?

Ans – Remote Desktop Protocol (RDP) is a proprietary protocol developed by Microsoft that allows users to remotely connect to another computer over a network connection securely using graphical interface capabilities.

- RDP enables remote management tasks such as accessing files or running applications as if you were physically present at that machine while providing encryption options for secure sessions during remote access operations.

Question 13 – What are some common troubleshooting steps for networking issues?

Ans – Troubleshooting networking issues typically involves these steps:

- Check physical connections: Ensure cables are plugged in correctly.

- Verify device configurations: Confirm settings like IP addresses match expected values.

- Use diagnostic tools: Employ commands like ping or traceroute to test connectivity.

- Restart devices: Power cycling routers/modems often resolves temporary glitches.

Following these steps systematically helps identify root causes effectively during troubleshooting processes.

Question 14 – What is a proxy server?

Ans – A proxy server acts as an intermediary between a client and the Internet. When a client requests resources from the Internet, the request first goes to the proxy server, which then forwards it to the destination server.

- The response from the destination server is sent back to the proxy, which forwards it to the client.

- Proxy servers can provide several benefits, including improved security by hiding the client’s IP address, caching content to reduce load times for frequently accessed resources, and filtering traffic to block access to certain websites or content types.

- They are commonly used in corporate environments to enforce security policies and manage Internet usage.

Question 15 – What is a network interface card (NIC)?

Ans – A Network Interface Card (NIC) is a hardware component that allows a device to connect to a network.

- It can be wired (using Ethernet) or wireless (using Wi-Fi). The NIC has a unique identifier known as a MAC address, which helps in identifying devices on a local network.

- NICs can be integrated into the motherboard or added as separate expansion cards.

- They play a crucial role in facilitating communication between devices on a network by converting data into signals that can be transmitted over various media.

Question 16 – What is bandwidth throttling?

Ans – Bandwidth throttling is the intentional slowing down of Internet speed by an Internet Service Provider (ISP) or network administrator.

- This practice can be applied to specific types of traffic or users to manage overall network performance and ensure fair usage among all users.

- For example, an ISP might throttle streaming services during peak hours to prevent network congestion.

- While it helps maintain service quality for all users, it can frustrate those who rely on high-speed connections for activities like gaming or video conferencing.

Question 17 – What is a DMZ in networking?

Ans – A Demilitarized Zone (DMZ) in networking refers to a physical or logical subnetwork that contains and exposes an organization’s external-facing services to an untrusted network, typically the Internet.

- The DMZ acts as a buffer zone between the internal network and external networks, enhancing security by isolating sensitive internal systems from direct exposure.

- Common services placed in a DMZ include web servers, email servers, and DNS servers.

- By placing these services in a DMZ, organizations can protect their internal networks from potential attacks while still providing necessary access for external users.

Question 18 – What is an intrusion detection system (IDS)?

Ans – An Intrusion Detection System (IDS) monitors network traffic for suspicious activity or policy violations and alerts administrators when such events occur. IDS can be classified into two main types:

- Network-based IDS (NIDS): Monitors traffic across entire networks.

- Host-based IDS (HIDS): Monitors individual devices or hosts for suspicious activity.

IDS plays a critical role in enhancing network security by identifying potential threats before they can cause significant damage.

Question 19. What is port scanning?

Ans – Port scanning is a technique used to identify open ports and services available on a networked device.

- It involves sending requests to various ports on a target system and analyzing responses to determine which ports are open, closed, or filtered.

- Port scanning is often used by administrators for security assessments but can also be exploited by attackers seeking vulnerabilities in systems.

- Tools like Nmap are commonly used for performing port scans.

Question 20 – What is IPv6?

Ans – IPv6 (Internet Protocol version 6) is the most recent version of the Internet Protocol designed to replace IPv4 due to its limited address space.

- IPv6 uses 128-bit addresses, allowing for an almost limitless number of unique IP addresses compared to IPv4’s 32-bit addressing scheme.

- The transition from IPv4 to IPv6 addresses issues like address exhaustion while providing improved routing efficiency and enhanced security features built directly into the protocol.

Question 21 – What does BYOD mean?

Ans – BYOD stands for “Bring Your Own Device,” referring to policies allowing employees to use their personal devices – like smartphones, tablets, or laptops – for work purposes within an organization’s network.

- While BYOD enhances flexibility and productivity, it also raises security concerns regarding data protection and compliance with organizational policies.

- Effective management strategies must be implemented to safeguard sensitive information accessed through personal devices.

More Articles

- Operating System Interview Questions

- Python Interview Questions

- DBMS Interview Questions

- Node.js Interview Questions

Interview Preparation

Prepare for Product/Service Based

Prime Course Trailer

Related Banners

Get PrepInsta Prime & get Access to all 200+ courses offered by PrepInsta in One Subscription

Get Prime

Company Wise Interview Questions

Still Thinking?

Choose Best Company Wise Interview Preparation Course on PrepInsta Prime

Get! Set! Click

80% Off

TCS Computer Network Questions

Here are 10 detailed Computer Network questions and answers frequently asked by TCS during technical interviews:

Q1. What is a Computer Network?

A computer network is a group of connected devices (like computers, printers, mobile phones) that can communicate and share data or resources with each other.

- These devices are linked using cables, Wi-Fi, or other transmission media. For example, if two computers are connected in an office and can share files or use the same printer, they form a computer network.

- It improves communication, efficiency, and reduces costs by sharing resources. In interviews, this question checks your understanding of basic networking.

Q2. What is the difference between LAN, MAN, and WAN?

- LAN (Local Area Network) connects devices within a small area, like a home, school, or office. It’s fast and usually privately owned.

- MAN (Metropolitan Area Network) covers a larger area like a city and is used by government or private organizations.

- WAN (Wide Area Network) covers even broader areas like countries or continents, with the Internet being the best example. TCS expects you to understand which type of network suits different scenarios.

Q3. What is an IP address?

An IP address is a unique numerical label assigned to each device connected to a network. It helps in identifying and locating devices.

- Just like you have a home address to receive mail, computers use IP addresses to send and receive data.

- There are two types: IPv4 (e.g., 192.168.0.1) and IPv6 (a newer version with a longer format).

Q4. What is the difference between TCP and UDP(TCP vs UDP)?

TCP (Transmission Control Protocol) is connection-oriented, reliable, and ensures data reaches the destination in order. It’s used in emails, websites, etc.

UDP (User Datagram Protocol) is connectionless, faster but less reliable, used in live streaming or online gaming. In interviews, explaining real-life examples helps show practical understanding.

Q5. What is OSI model?

OSI (Open Systems Interconnection) model is a 7-layer framework that shows how data travels from one device to another. The layers are: Physical, Data Link, Network, Transport, Session, Presentation, and Application. Each layer has a specific role in transmitting data. For example, the Transport layer ensures data is sent correctly, while the Application layer allows users to access the network (e.g., using a browser). Interviewers expect you to explain the purpose of each layer.

Q6. What happens when you type www.google.com in a browser?

When you type a URL like www.google.com, your computer contacts a DNS server to find Google’s IP address.

- Then it creates an HTTP request using the TCP/IP model and sends it over the Internet.

- The request goes through different layers of networking and reaches Google’s server.

- The server responds, and you see the webpage. This question tests your understanding of the full networking process.

Q7. What is a switch?

A switch is a device used in LANs to connect multiple devices. It uses MAC addresses to send data only to the intended device, improving speed and security. Unlike hubs that broadcast data to all devices, switches are smarter and more efficient. TCS may ask this to see if you understand basic networking hardware.

Q8. What is a router?

A router connects different networks together. At home, it connects your personal devices to the internet. In offices, routers manage traffic between different LANs. It uses IP addresses to forward data to the correct network. Routers are essential in any network setup, and knowing their function is key in interviews.

Q9. What is DNS?

- DNS (Domain Name System) converts domain names like www.google.com into IP addresses like 142.250.77.132.

- This way, humans can use easy-to-remember names, and computers can still communicate using IPs.

- It works like a phonebook for the internet. Without DNS, we’d have to memorize numerical IPs for every website.

PS – Interviewers check your understanding of how name resolution works.

Q10. What is a MAC address?

A MAC (Media Access Control) address is a hardware address assigned to a device’s network interface card. It is used in the Data Link Layer to identify devices on a local network.

- Unlike IP addresses, MAC addresses are permanent and unique to each device. Knowing the difference between MAC and IP addresses is crucial for network troubleshooting.

Infosys Frequently Asked Questions

Here are 10 elaborative questions and answers on Computer Networks asked by Infosys:

Q1. What is the use of the Data Link Layer?

The Data Link Layer is the second layer in the OSI model. Its main job is to transfer data from one node to another over the same network.

- It also checks for errors in transmission and controls how data is placed and received on the medium. It uses MAC addresses to identify devices.

- For instance, if two computers are connected in a LAN, the Data Link Layer ensures the data goes to the right computer without errors.

- Infosys asks this to test if you understand practical layer functions.

Q2. What is a Firewall?

A firewall is a security device or software that filters incoming and outgoing network traffic based on rules. It helps protect your system or network from unauthorized access, viruses, and attacks. Think of it as a security guard checking who is allowed in and out. In interviews, you can explain how firewalls are used in both home and corporate networks.

Q3. What is the use of DHCP?

DHCP (Dynamic Host Configuration Protocol) automatically assigns IP addresses to devices when they join a network.

- Without it, each IP would have to be set manually. DHCP makes network management easier, especially in offices with many devices. Infosys expects you to know how dynamic addressing works.

Q4. What is Subnetting?

Subnetting is dividing a network into smaller parts called subnets. It helps organize networks efficiently, reduce congestion, and enhance security. For example, a company may subnet different departments to isolate their data. It also helps in better IP address management. This topic shows up in network design-related questions.

Q5. What is the difference between HTTP and HTTPS?

HTTP (HyperText Transfer Protocol) is used for transferring data on the web.

HTTPS is the secure version of it. The ‘S’ stands for SSL/TLS encryption, ensuring data between browser and server is safe from hackers.

Q6. What is the function of the Transport Layer?

The Transport Layer ensures reliable data transfer between systems. It breaks data into smaller segments, sends them, and reassembles them at the destination. It uses TCP for reliable communication and UDP for faster but less secure transfer. Understanding this layer is essential in troubleshooting network issues.

Q7. What is Packet Switching?

- Packet switching breaks data into packets, sends them through different paths in the network, and reassembles them at the destination.

- It’s used in the Internet because it’s fast, efficient, and resilient to failures.

- Infosys values this concept due to its real-world application.

Q8. What is the use of ARP?

ARP (Address Resolution Protocol) maps IP addresses to MAC addresses within a LAN. When a device wants to send data, it uses ARP to find the physical address associated with the IP. It is important for local communication between devices.

Q9. What is Bandwidth?

Bandwidth refers to the data transfer capacity of a network, usually measured in Mbps or Gbps. Higher bandwidth means faster data transfer. For example, streaming HD videos smoothly requires higher bandwidth. Understanding bandwidth is essential in network planning.

Q10. What is Latency?

Latency is the delay before a transfer of data begins following an instruction. It is measured in milliseconds. Low latency means faster communication, which is important for video calls, gaming, and real-time apps. Infosys checks this to see if you understand performance factors in networking.

Final Thoughts

Preparing for computer network interview questions can be overwhelming, but with the right resources and practice, you can confidently tackle any question. This blog has covered 50+ essential networking interview questions to help you understand the key concepts and prepare effectively. Remember to practice explaining these concepts in simple terms, as clarity is key during interviews.

Good luck with your interview preparation! If you found this blog helpful, feel free to share it with others who might benefit from it. Happy learning!

FAQs - Computer Network Interview Questions

How does the TCP three-way handshake work?

Explain how congestion control works in TCP.

What is the difference between Circuit Switching and Packet Switching?

How does NAT (Network Address Translation) work?

What is the difference between Layer 2 and Layer 3 switches?

Get over 200+ course One Subscription

Courses like AI/ML, Cloud Computing, Ethical Hacking, C, C++, Java, Python, DSA (All Languages), Competitive Coding (All Languages), TCS, Infosys, Wipro, Amazon, DBMS, SQL and others

Get Prime

Course List

Checkout list of all the video courses in PrepInsta Prime Subscription